- Give the functional form and sketch both the real and imaginary components of the Fourier transformation (with respect to x) of the following functions. Identify the spatial frequencies, wave-numbers,  and features in the spectra. ko is a constant real number					(25 points)

  - coskox+isinkox						(5 points)

  - TopHatx4							(5 Points)

c. sin8koxcoskox						(5 points)

d. n=-22dx−n=n=-¥¥dx−n·TopHatx2			(5 Points)

e. TopHat(x−4)							(5 Points)

- Show that the k=0 point of F(k) is equal to the area f(x), where f(x)  F(k). 										(10 points)

- Show that the kx=0 point of Fkx,ky is equal to the projection of fx,y onto the y-axis where fx,yÛFkx,ky.				(5 points)

- A simple way of characterizing the spatial distribution of a radiation source is to image it with a pin-hole imager.				(40 points)

  - Draw the experimental geometry, and explain why this is a useful experiment. What is the form of the image, Ix,y, in terms of the source distribution, Sx,y, assuming a perfect pin-hole camera? Place the a distance, a, from the source and the screen (detectors) and a distance, b,  from the pin-hole. Include the magnification in your answer. 							(20 points)

  - Assume the pin-hole is of a finite size (radius = r), and therefore a true representation of the source is not observed. In linear imaging terms, explicitly describe the detected signal, Ix,y, in both the source distribution and the pin-hole size.				(10 points)

  - There is an oblique effect if the source extends far from the central line through the pin-hole. Without calculating the geometry of this effect, describe how it arises.				(10 points)

- The Nyquist condition states that to correctly measure a frequency the signal must be sampled at least twice a period.				(20 points)

  - Let f be the Nyquist frequency, show that the signals, cos2pf+Dftand cos2pf−Dft,  lead to the exact same data points when sampled at times tn=n2f.				(10 points)

  - Explain aliasing in terms of the above result.		(10 points)